;*************************************************************

;**** Program title: HW3.1 COPY 20H-2FH to 30H-3FH ***********

;**** Programmer: SUPPAKORN HENGPRASITH 5913370    ***********

;*************************************************************

PROCESSOR PIC16F628

#include <P16F628.INC>

__CONFIG	_CP_OFF & _MCLRE_ON & _INTRC_OSC_NOCLKOUT & _LVP_OFF & _WDT_OFF

;Declare File register

counter	EQU	40H

value	EQU	41H

source	EQU 42H

destination	EQU	43H

ORG	0x00	; reset vector

GOTO START	; jump to start of the program

ORG	0x04	; Interrupt vector

START:

; SET 20-2F to 0-F

movlw	20H

movwf	FSR

clrw			; clrw = movlw 0H

movwf	value	; set value = w = 0

movlw	.16		; set w = 16

movwf	counter	; set counter =16

AGAIN1:  movf	value,w

movwf	INDF

incf	value,f

incf	FSR,f

decfsz	counter,f

goto	AGAIN1

; HW3 Move 20-2F to 30-3F

movlw	.16		; set w = 16

movwf	counter	; set counter =16

movlw	20H		; set w = 20H

movwf	source	; set source = 20H

movlw 	30H		; set w = 30H

movwf 	destination	; set destination = 30H

AGAIN2:

movf	source,w

movwf	FSR

movf	INDF,w

movwf	value

movf	destination,w

movwf	FSR

movf 	value,w

movwf	INDF

incf	source,f

incf	destination,f

decfsz	counter,f

goto 	AGAIN2

END

;*******************************************

;HW32 2switch2led toggle

;*******************************************

PROCESSOR PIC16F628

#include <P16F628.INC>

__CONFIG	_CP_OFF & _MCLRE_ON & _INTRC_OSC_NOCLKOUT & _LVP_OFF & _WDT_OFF

;Declare File register

SW1_STATE	EQU	40H

SW2_STATE	EQU 41H

ORG	0x00	; reset vector

GOTO START	; jump to start of the program

ORG	0x04	; Interrupt vector

START:	; Port Configuration

MOVLW	.7

MOVWF	CMCON		; CMCON=7 turn-off analog comparator inputs

BCF		STATUS,RP1	; or 'BCF 0x03,0x06'

BSF		STATUS,RP0	; Select Bank1

BSF		TRISA,0		; Port RA0 is an Input pin

BSF		TRISA,1		; Port RA1 is an Input pin

BCF		TRISB,0		; Port RB0 is an output pin

BCF		TRISB,1		; Port RB1 is an output pin

BCF		STATUS,RP0	; Back to Bank0

; Start your program here

; K1 ACTIVE HIGH K2 ACTIVE LOW

; LED1 ACTIVE HIGH LED2 ACTIVE LOW

CLRW

MOVWF	SW1_STATE	;set SW1_STATE=0

MOVWF	SW2_STATE	;set SW2_STATE=0

Inf_loop:

BTFSC	PORTA,1		; K1 ACTIVE HIGH use BTFSC

GOTO 	TOGGLE_SW1	;

RETURN_TOGGLE_SW1:

BTFSS 	PORTA,0		; K2 ACTIVE LOW use BTFSS

GOTO	TOGGLE_SW2	;

RETURN_TOGGLE_SW2:

BTFSC	SW1_STATE,0

GOTO	LED1_ON

GOTO	LED1_OFF

RETURN_LED1_ONOFF:

BTFSC	SW2_STATE,0

GOTO	LED2_ON

GOTO	LED2_OFF

LED1_ON:

BSF		PORTB,1		; LED1 ACTIVE HIGH >> on use BSF

GOTO 	RETURN_LED1_ONOFF

LED1_OFF:

BCF		PORTB,1		; LED1 ACTIVE HIGH >> off use BCF

GOTO 	RETURN_LED1_ONOFF

LED2_ON:

BCF		PORTB,0		; LED2 ACTIVE LOW >> on use BCF

GOTO	Inf_loop

LED2_OFF:

BSF		PORTB,0		; LED2 ACTIVE LOW >> off use BSF

GOTO	Inf_loop

TOGGLE_SW1:

MOVLW	.1

XORWF	SW1_STATE,f	; TOGGLE SW1_STATE

GOTO	RETURN_TOGGLE_SW1

TOGGLE_SW2:

MOVLW 	.1

XORWF	SW2_STATE,f	; TOGGLE SW2_STATE

GOTO	RETURN_TOGGLE_SW2

END